Description

- Combined compact potwashing and dishwashing machine with frontal door opening that can be opened to 90° or 180°, and a rotating wash basket.

- Wash technology with low water pressure for dishwashing and higher water pressure with or without granule blasting for washing of pots, pans and utensils. Solution that allows for savings in time, water, electricity and chemical consumption as well as eliminating the need for soaking, scrubbing and manual pre-washing of cookware.

- Operating panel at eye level and within easy reach. Waterproof backlit screen with alphanumeric display. Count-down and indicator light showing program end. Preventive alerts: Change of wash water, change of Granules, time for service.

- Energy-saving mode if not used for 20 min.

- Unique QR code for online access to user manuals, servicing history, and troubleshooting guides.

- HACCP traceability via USB 2.0 port equipped with waterproof lid. 200 latest wash program parameters can be exported.

Productivity

- 3 wash modes with automatic activation by detection of the wash basket to prevent any dishware washing with cookware high pressure pump.

- 9 wash programs: from 1:30 min to 5:30 min. - Cleaning of wash cabin and granule collection program.

- Capacity per wash cycle: 6 GN 1/1 or GN 1/2 depth 65 mm, 3 GN 1/1 depth 200 mm and 3 GN 1/1 depth 65 mm, 18 dining plates or 12 soup plates Ø 240 mm, 25 mugs Ø 90 mm.

Select one of the two following versions:

  Art.27023 - Compact version to wash GN, pots and dishes

- Hourly productivity: 108 GN 1/1 or 33 dishware wash basket size 500x500 mm.

- Standard delivery: 1 Cookware wash basket special GN and pots with bottom grid, 1 Dishware wash basket and its cover grid, 1 Granule collector, 1 Starter load of granules 8 litres, 1 Scraper, installation and user manuals, instruction poster.

  Art.27025 - High productivity version with trolleys, to wash GN, pots and dishes

- Hourly productivity: 126 GN 1/1 or 33 dishware wash basket size 500x500 mm.

- Standard delivery: 2 Cookware wash basket special GN and pots with/without bottom grid, 2 trolleys for cookware wash basket, 1 Dishware wash basket and its cover grid, 1 Granule collector, 1 Starter load of granules 8 litres, 1 Scraper, installation and user manuals, instruction poster.

Specifications and compliance

- Wash tank 108 litres. Heating element 14 kW. Wash temperature 65 °C. 4 Fixed wash arms. 24 polymer wash nozzles removable without tools designed to optimize the lifespan of granules. High pressure pump 2.2 kW. Low pressure pump 0.7 kW.

- Rinse tank 7.5 litres. Heating element 14 kW. Rinse temperature 85 °C. Rinse arm with 12 nozzles. Rinse pump 0.11 kW.

- Consumption of water per wash program: Minimum 4 litres.

- EN 1.4301 Stainless steel design: Frame, door, impeller, tanks, wash and rinse arms. Heat and noise insulation.

- Dimensions: W 1025 x D 957 x H 1736/2079 (± 25) mm. Weight: Net 331 kg - Gross 465 kg.

- Sound pressure level: 70.0 dB(A) ISO 11203

- Heat dissipation: Total 2.4 kW - Sensible 0.8 kW - Latent 1.6 kW.

- Norms and certification compliance IPX5 - CE - DIN 10534 - EN 1717 - RoHS.

Installation

- Electrical: 32 A - 17.2 kW - 3 AC 400-415 V - 50 Hz. Supplied with 2 metres long power cable to connect to an approved mains switch.

- Hot or cold water softened inlet: 5-65 °C - 3-7 °dH (5-12 °fH) - 53-125 PPM - 1-6 bar - 15 litres/min - ¾” male (DN 20).

- Drain: Ø 32 mm - 65 °C - 50 litres/min.

- Chemicals: 230 V built-in electrical connection box. Wash tank prepared for a detergent probe installation.

Factory fitted options

- Efficient steam condensing unit with heat recovery system. Saves up to 40% of the energy required to heat the rinse tank. Connection to 5-20 °C cold water inlet. Extends the duration of the wash program up to 80 sec. Total height of machine increased to 1964 (± 25) mm. Heat dissipation: Total: 1.3 kW - Sensible: 0.8 kW - Latent: 0.5 kW.

- Built-in unit condensing steam using cold water. Water inlet: 5-20 °C - 53-125 ppm - 3-6 bar - 15 litres/min - 
¾” male (DN 20). Extends the duration of wash program up to 70 sec. Heat dissipation: Total: 1.7 kW - 
Sensible: 0.8 kW - Latent: 0.9 kW.

- Integrated liquid detergent and rinse aid dosing dispensers with peristaltic pump. Suction hoses. Detergent dosing sensor installed in the wash tank.

- Interface for energy optimization. Relay terminal block that can be controlled by dry contact or 230 V.

- Power supply versions and other voltages - Hot-water softened inlet:

 32 A - 17.2 kW - 3 AC 380 V - 50 Hz

 32 A - 17.2 kW - 3 AC 400-415 V - 60 Hz

 32 A - 17.2 kW - 3 AC 380 V - 60 Hz

- Marine version: 2 level sensors in the wash tank to compensate ship’s movement. Feet with floor anchoring system. Service access from the front.

Options

- Non-electric water softener for hot and cold water. Dual tank. 4 castors. Regeneration at 2 bar 24 litres/11 min.  Salt capacity 18 kg. Dimensions: L 356 x W 356 x H 559 mm. Water inlet Ø 3/4” - 2-65 °C - 2.5-8.5 bar. Drain connection 0.375’’.

- Trolley for wash basket. 4 wheels with brakes. Dimensions: 800 x 870 mm.

- Holder for bowls and utensils. To be placed inside the wash basket. Capacity: Up to 5-6 bowls and ladles.

- Holder for 2 GN 2/1, 150 mm depth, or 16 GN 1/1 grids, or 8 lids GN 1/1 or 6 baking trays 600x400x20 mm. To be placed inside the wash basket.

- Pot basket. Holds 1 large pot (400 mm depth) or 2 large pots (200 mm depth). To be used as alternative to the regular wash basket.

- Net basket with lid to wash ladles, whisks up to 500 mm length.

- Multiflex holder for saucepans, small bowls, sieves. To be placed inside the wash basket. Capacity: up to 3-5 saucepans or small bowls, sieves.

- Flexible pot holder incl. 4 rubber straps and two hooks.

- Holder for storage of inserts and accessories not in use. To hang at either side of the pot washing machine.

- Set of 3 scrapers to eliminate food residues before loading of pots or pans in the pot washing machine.

- 10-litre canister of biodegradable granules made of rapeseed oil, calcium carbonate, and biodegradable polyesters.

- Dishware wash basket. Dimension: 500x500 mm. Capacity: 18 Dining plates or 12 soup plates Ø 240 mm.

- Glassware wash basket. Dimension: 500x500 mm. Capacity: 25 mugs Ø 90 mm.