Description

- Pot and pan washing machine with automatic hood and rotating wash basket. High flow and pressure wash technology that operates with or without blasting of granules, which allows for reduction of time, water, energy and detergent consumption while also eliminating the need for soaking and pre-washing cookware.

- Can be placed along a wall or in a corner.

- Operating panel at eye level and within easy reach. Waterproof backlit screen with alphanumeric display. Count-down and indicator light showing program end. Preventive alerts: Change of wash water, change of Granules, time for service.

- Energy-saving mode if not used for 20 min.

- Unique QR code for online access to user manuals, servicing history, and troubleshooting guides.

- HACCP traceability via USB 2.0 port equipped with waterproof lid. 200 latest wash program parameters can be exported.

Productivity

- 2 wash modes, granule-optimized or with water only. 6 wash programs: from 2:10 min to 5:20 min. - Cleaning of wash cabin and granule collection program.

- Capacity per wash cycle: 6 GN 1/1 or GN 1/2 65 mm deep. 3 GN 1/1 200 mm deep and 3 GN 1/1 65 mm deep.

- Hourly productivity 114 GN 1/1.

- Standard delivery: 1 Cookware wash basket special GN and pots with bottom grid, 1 Granule collector, 1 Starter load of granules 8 litres, 1 scraper, Installation and user manuals, instruction poster.

Specs and compliance

- Wash tank 83 litres. Heating element 9 kW. Wash temperature 65 °C. 2 fixed wash arms. 12 polymer wash nozzles removable without tools, designed to optimize the lifespan of granules. High pressure pump 2.6 kW.

- Rinse tank 10.5 l. Heating element 9 kW. Rinse temperature 85 °C. Rinse arm with 12 nozzles. Rinse pump 0.37 kW.

- Consumption of water per wash program: Minimum 4 litres.

- EN 1.4301 Stainless steel design: Frame, hood, tanks, wash and rinse arms. Heat and noise insulation.

- Dimensions: W 850 x D 1002 x H 1693/2339 (± 25) mm. Weight: Net 304 kg - Gross 403 kg.

- Sound pressure level: 69.7 dB(A) ISO 11201

- Heat dissipation: Total 1.6 kW - Sensible 0.4 kW - Latent 1.2 kW.

- Norms and certification compliance IPX5  CE  DIN 10534  EN 1717  RoHS.

Installation

- Electrical: 25 A - 11.9 kW - 3 AC 400-415 V - 50 Hz. 2 m. long power cable to connect to an approved mains switch.

- Hot-water softened inlet: 55-65 °C - 3-7 °dH (5-12 °fH) - 53-125 PPM - 1-6 bar - 15 litres/min - ¾” male (DN 20).

- Cold-water inlet: 5-20 °C - 3-7 °dH (5-12 °fH) - 53-125 PPM - 1-6 bar - 15 litres/min - ¾” male (DN 20).

- Drain: Ø 32 mm - 65 °C - 50 litres/min.

- Chemicals: 230 V built-in electrical connection box. Wash tank prepared for a detergent probe installation.

Factory fitted options

- Built-in unit condensing steam using cold water. Water inlet: 5-20 °C - 53-125 PPM - 3-6 bar - 15 litres/min - ¾” male (DN 20). Extends the duration of the wash program up to 30 sec. Heat dissipation: Total: 1.2 kW - Sensible: 0.4 kW - Latent: 0.8 kW.

- Integrated liquid detergent and rinse aid dosing dispenser with peristaltic pump. Suction hoses. Detergent dosing sensor installed in the wash tank.

- Interface for energy optimization. Relay terminal block that can be controlled by dry contact or 230 V.

- High-power supply version and other voltages - Cold or hot water softened inlet and cold water:

 32 A - 16.9 kW - 3 AC 400-415 V - 50 Hz

- Standard power supply versions and other voltages - Hot-water softened inlet and cold water:

 40 A - 11.9 kW - 3 AC 230 V - 50 Hz

Options

- Non-electric water softener for hot and cold water. Dual tank. 4 castors. Regeneration at 2 bar 24 litre/11 min. Salt capacity 18 kg. Dimensions: L 356 x W 356 x H 559 mm. Water inlet Ø 3/4” - 2-65 °C - 2.5-8.5 bar. Drain connection 0.375’’.

- Trolley for wash basket. 4 wheels with brakes. Dimensions: 800 x 870 mm.

- High-capacity package incl. 1 cookware wash basket special GN and pots with bottom grid and 1 holder to place inside the wash basket for 2 GN 2/1, 150 mm depth or 16 grids GN 1/1 or 8 lids GN 1/1 or 6 baking trays 600x400x20 mm.

- Holder for bowls and utensils. To place inside the wash basket. Capacity: Up to 5-6 bowls and ladles.

- Holder for 2 GN 2/1, 150 mm depth or 16 racks GN 1/1 or 8 lids GN 1/1 or 6 pastry trays 600x400x20 mm. To place inside the wash basket.

- Pot basket. Holds 1 large pot (400 mm depth) or 2 large pots (200 mm depth). To use as an alternative to the regular wash basket.

- Net basket with lid to wash ladles, whisks up to 500 mm long.

- Multiflex holder for saucepans, small bowls, sieves. To be placed inside the wash basket. Capacity: up to 3-5 saucepans or small bowls, sieves.

- Flexible potholder incl. 4 rubber straps and two hooks.

- Set of 3 scrapers to eliminate food residue before loading pots or pans into the pot washing machine.

- 10-litre canister of biodegradable granules made of rapeseed oil and biodegradable polyesters.

- Outlet folding table. Can be installed on front or on either side. Dimension: 780 x 780 mm.