Description

- Combined potwashing and dishwashing machine with automatic hood and rotating wash basket. Low pressure wash technology for dishwashing and high flow and pressure wash technology that operates with or without the blasting of granules for washing pots and pans, which allows for reduction of time, water, energy, and detergent consumption while also eliminating the need for soaking and pre-washing cookware.

- Can be placed along a wall or in a corner.

- Operating panel at eye level and within easy reach. Touch screen with pictograms enabling easy and intuitive operations. Preventive alerts: Change of wash water, change of Granules, time for service.

- Unique QR code for online access to user manuals, servicing history, and troubleshooting guides.

- HACCP traceability via USB 2.0 port equipped with waterproof lid. 1000 latest wash program parameters can be exported.

- Standard delivery: 1 Pro Wash basket, 1 Utensils Basket. 1 Dishwash Basket, 1 Starter load of granules 7 litres, 
1 Scraper.  Installation and user manuals, instruction poster.

Productivity

- 3 wash modes with automatic activation by detection of the wash basket to prevent any dishware washing with cookware high pressure pump.

- 6 wash programs: from 1:00 min to 4:15 min. - Auto granule collection and auto clean of granules and machine.

- Capacity per wash cycle: 7 GN 1/1 or GN 1/2 65 mm deep or 3 GN 1/1 200 mm deep and 3 GN 1/1 65 mm deep, 18 dining plates or 12 soup plates Ø 240 mm, 25 mugs Ø 90 mm.

- Hourly productivity: 126 GN 1/1 or 40 dishware wash basket size 500x500 mm.

Specifications and compliance

- Wash tank 80 litres. Heating element 14 kW. Wash temperature 65 °C. 2 fixed wash arms and 2 removable wash arms. 12 polymer wash nozzles, designed to optimize lifespan of granules. High pressure pump 2.2 kW. 13 precut nozzles for dish washing. Low pressure pump 0.7 kW.

- Rinse tank 12 l. Heating element 14 kW. Rinse temperature 85 °C. Rinse arm with 12 nozzles. Rinse pump 0.11 kW.

- Consumption of water per wash program: 3.6 litres.

- EN 1.4301 Stainless steel design: Frame, hood, tanks, wash and rinse arms.

- Heat and noise insulation.

- Dimensions: W 849 x D 990 x H 1695/2365 (± 35) mm. Weight: Net 322 kg - Gross 412 kg.

- Sound pressure level: 65.9 dB(A) ISO 11203

- Heat dissipation: Total 1.6 kW - Sensible 0.4 kW - Latent 1.2 kW.

- Norms and certification compliance IPX5 - CE - EN 17735:2023 - EN 1717 - RoHS.

Installation

- Electrical: 32 A – 17.5 kW - 3 AC 400-415 V - 50 Hz. 2 m long power cable to connect to an approved mains switch.

- Hot-water softened inlet: 55-65 °C - 3-7 °dH (5-12 °fH) - 53-125 PPM - 1-6 bar - 15 litres/min - ¾” male (DN 20).

- Cold-water inlet: 5-20 °C - 3-7 °dH (5-12 °fH) - 53-125 PPM - 1-6 bar - 15 litres/min - ¾” male (DN 20).

- Drain: Ø 32 mm - 65 °C - 50 litres/min.

- Chemicals: 230 V built-in electrical connection box.  Wash tank prepared for a detergent probe installation.

Factory fitted options

- Efficient steam condensing unit with heat recovery system. Saves up to 40% of the energy required to heat the rinse tank. Connection to 5-20 °C cold water inlet. Extends the duration of the wash program up to 80 sec. Total height of machine increased to 1964 (± 25) mm. Heat dissipation: Total: 0.9 kW - Sensible: 0.4 kW - Latent: 0.5 kW.

- Integrated liquid detergent and rinse aid dosing dispenser with peristaltic pump. Suction hoses. Detergent dosing sensor installed in the wash tank.

- Interface for energy optimization. Relay terminal block that can be controlled by dry contact or 230 V.

Options

- Non-electric water softener for hot and cold water. Dual tank. 4 castors. Regeneration at 2 bar 24 litres/11 min. Salt capacity 18 kg. Dimensions: L 356 x W 356 x H 559 mm. Water inlet Ø 3/4” - 2-65 °C - 2.5-8.5 bar. Drain connection 0.375’’.

- Trolley for wash basket. 4 wheels with brakes. Dimensions: 800 x 870 mm.

- Multiflex holder for saucepans, small bowls, sieves. To be placed in the wash basket. Capacity: up to 3-5 saucepans or small bowls, sieves.

- Flexible potholder incl. 4 rubber straps and two hooks.

- Set of 3 scrapers to eliminate food residue before loading pots or pans into the pot washing machine.

- 10-litre canister of biodegradable granules made of rapeseed oil and biodegradable polyesters.

- Glassware wash basket. Dimension: 500x500 mm. Capacity: 25 mugs Ø 90 mm.