Description

- Compact front-loaded pot and pan washing machine. High flow and pressure wash technology that operates with or without blasting of granules, which allows for reduction of time, water, energy and detergent consumption while also eliminating the need for soaking and pre-washing cookware.

- Operating panel at eye level and within easy reach. Waterproof backlit screen with alphanumeric display. Count-down and indicator light showing program end. Preventive alerts: Change of wash water, change of granules, time for service.

- Energy-saving mode if not used for 20 min.

- Unique QR code for online access to user manuals, servicing history, and troubleshooting guides.

- HACCP traceability via USB 2.0 port equipped with waterproof lid. 200 latest wash program parameters can be exported.

- Built-in steam condensing unit using cold water. Extends the duration of the wash program up to 60 sec.

Productivity

- 2 wash modes. 6 wash programs: from 2:40 min to 6:50 min. Cleaning of wash cabin and granule collection program.

- Capacity per wash cycle:

  - 12 GN 1/1 up to 65 mm or 8 GN 1/1 200 mm

  - 6 GN 2/1 up to 65 mm or 4 GN 2/1 200 mm

  - 12 GN 1/2 up to 200 mm with 2 dividers (19313, not included) or 4 GN 1/2 up to 200 mm without divider.

Select one of the three following versions:

  - Art.20721 - Compact with sliding cassette

- Hourly productivity: 168 GN 1/1

- Standard delivery: 1 ProCassette, 1 Divider to load GN 2/3 up to 1/9 depth 200 mm, 2 Granule collectors, 1 Starter load of Granules 16 l., 1 scraper, installation and user manuals, instruction poster.

  - Art.20723 – High productivity - with 2 cassettes and 2 trolleys

- Hourly productivity: 228 GN 1/1

- Standard delivery: 2 ProCassettes, 2 Dividers to load GN 2/3 up to 1/9 depth 200 mm, 2 Trolleys and drip trays, 
2 Granule collectors, 1 Starter load of granules 16 l., 1 scraper, installation and user manuals, instruction poster.

  - Art.20722 - 2 cassettes, hooks with wall protection and 1 trolley

- Hourly productivity: 204 GN 1/1

- Standard delivery: 2 ProCassettes, 2 Dividers to load GN 2/3 up to 1/9 depth 200 mm, 2 sets of Cassette hooks with wall protection, 1 Trolley and drip tray, 2 Granule collectors, 1 Starter load of Granules 16 l., 1 scraper, installation and user manuals, instruction poster.

Specifications and compliance

- Wash tank 165 litres. Heating element 16 kW. Wash temperature 65 °C. 4 oscillating, motorized wash arms removable without tools. 36 removable polymer wash nozzles designed to optimize the lifespan of the granules. 2 High pressure pumps 2 x 2.2 kW.

- Rinse tank 16 litres. Heating element 16 kW. Rinse temperature 85 °C. Rinse pipes with 20 nozzles. Rinse pump 0.37 kW.

- Consumption of water per wash program: Minimum 7 litres.

- EN 1.4301 Stainless steel design: Frame, doors, impeller, tanks, wash and rinse arms. Heat and noise insulation.

- Dimensions: W 1452 x D 1075 x H 2039 (± 25) mm. Weight: Net 485 kg - Gross: 666 kg.

- Sound pressure level: 69.8 dB(A), ISO 11203

- Heat dissipation: Total 2.7 kW - Sensible 1.0 kW - Latent 1.7 kW.

- Norms and certification compliance IPX5 - CE - DIN 10534 - EN 1717 - RoHS.

Installation

- Electrical: 40 A - 21 kW - 3 AC 400-415 V - 50 Hz. 2 m long electric cable to connect to an approved mains switch.

- Hot-water softened inlet: 55-65 °C - 3-7 °dH (5-12 °fH) - 53-125 PPM - 1-6 bar - 25 litres/min - ¾” male (DN 20).

- Cold-water inlet: 5-20 °C - 3-7 °dH (5-12 °fH) - 53-125 PPM - 3-6 bar - 15 litres/min - ¾” male (DN 20).

- Drain: Ø 32 mm - 65 °C - 50 litres/min.

- Chemicals: 230 V built-in electrical connection box. Wash tank prepared for a detergent probe installation.

Factory fitted options

- Doorway 900 mm: The machine is delivered with parts possible to disassemble (doors and front gutter) in order to reduce its size to 890 mm to pass through a standard door of 900 mm usable width. The parts must be reassembled and siliconed before the first use.

- Efficient steam condensing unit with heat recovery system. Saves up to 35% of the energy required to heat the rinse tank. Requires the high-power supply option. Cold water inlet connection 5-20 °C - 3-7 °dH (5-12 °fH) - 53-125 PPM - 3-6 bar - 15 litres/min - ¾” male (DN 20). Extends the duration of the wash program up to 120 sec. Total height of the pot washing machine increased to 2267 (± 25) mm. Heat dissipation: Total: 2 kW - Sensible: 1 kW - Latent: 1.1 kW.

- Integrated liquid detergent and rinse aid dosing dispenser with peristaltic pump. Suction hoses. Detergent dosing sensor installed in the wash tank.

- Interface for energy optimization. Relay terminal block that can be controlled by dry contact or 230 V.

- High-power supply version and other voltages - Cold or hot water softened inlet and cold-water inlet:

 3 AC 400-415 V - 50 Hz - 50 A - 29 kW

 3 AC 380 V - 50 Hz - 50 A - 29 kW

 3 AC 400-415 V - 60 Hz - 50 A - 29 kW

 3 AC 380 V - 60 Hz - 50 A - 29 kW

- Standard power supply versions and other voltages - Hot-water softened and cold-water inlet:

 3 AC 230 V - 50 Hz - 63 A - 21 kW

 3 AC 230 V - 60 Hz - 63 A - 21 kW

 3 AC 220 V - 60 Hz - 63 A - 21 kW

 3 AC 440 V - 60 Hz - 40 A - 21 kW

 3 AC 208 V - 60 Hz - 63 A - 21 kW

 3 AC 575 V - 60 Hz - 31 A - 21 kW

- Marine version: 2 level sensors in the wash tank to compensate for ship’s movement. Feet with floor anchoring system. Doorstops. Service access from the front.

Options

- Non-electric water softener for hot and cold water. Dual tank. 4 castors. Regeneration at 2 bar 24 litres/11 min. 
Salt capacity 18 kg. Dimensions: L 356 x W 356 x H 559 mm. Water inlet Ø 3/4” - 2-65 °C - 2.5-8.5 bar. 
Drain connection 0.375’’.

- Net basket with lid to wash ladles, whisks up to 500 mm long.

- Divider to load GN 2/3 up to 1/9 depth 200 mm.

- Lid and tray holder. Capacity: 9 pieces of GN 1/1 sized trays, lids, grids, baking trays.

- Holder for smaller bowls and pots.

- Utensils holder for ladles, whisks up to 500 mm in length.

- Holder for 6 GN 1/3 or 12 GN 1/6 or 18 GN 1/9.

- Insert for 8 baking or pastry trays or racks, sized 600 x 400 with an edge 20 mm.

- Holder to store inserts and accessories not in use. Hung at either side of the pot washing machine.

- Set of 3 scrapers to eliminate food residue before loading pots or pans into the pot washing machine.

- 10-litre canister of biodegradable granules made of rapeseed oil, calcium salt, and biodegradable polyesters.