Description

- Compact pot and pan washing machine that is front-loaded with door with double opening radius (90º and 180º) and rotating wash basket.

- High flow and pressure wash technology that operates with or without blasting of granules, which allows for reduction of time, water, energy and detergent consumption while also eliminating the need for soaking and pre-washing cookware.

- Operating panel at eye level and within easy reach. Waterproof backlit screen with alphanumeric display. Count-down and indicator light showing program end. Preventive alerts: Change of wash water, change of granules, time for service.

- Energy-saving mode if not used for 20 min.

- Unique QR code for online access to user manuals, servicing history, and troubleshooting guides.

- HACCP traceability via USB 2.0 port equipped with a waterproof lid. 200 latest wash program parameters can be exported.

Productivity

- 2 wash modes, granule-optimized or with water only. 6 wash programs: from 2:10 min to 5:10 min. Cleaning of wash cabin and granule collection program.

- Capacity per wash cycle: 6 GN 1/1 or GN 1/2 65 mm deep, 3 GN 1/1 200 mm deep and 3 GN 1/1 65 mm deep.

Select one of two versions:

 Art.24772 - Compact to wash GN and pots

- Hourly productivity: 126 GN 1/1

- Standard delivery: 1 Cookware wash basket special GN and pots with bottom grid, 1 Granule collector, 
1 Starter load of granules 8 litres, 1 Scraper, Installation and user manuals, and instruction poster.

 Art.26544 - High productivity - with trolleys, to wash GN and pots

- Hourly productivity: 144 GN 1/1

- Standard delivery: 2 Cookware wash basket special GN and pots with bottom grid, 2 Trolleys for cookware wash basket, 1 Granule collector, 1 Starter load of granules 8 litres, 1 Scraper, installation and user manuals, instruction poster.

Specifications and compliance

- Wash tank 108 litres. Heating element 9 kW. Wash temperature 65 °C. 2 Fixed wash arms. 12 polymer wash nozzles removable without tools designed to optimize the lifespan of granules. High-pressure pump 2.2 kW.

- Rinse tank 7.5 litres. Heating element 9 kW. Rinse temperature 85 °C. Rinse arms with 12 nozzles. 
Rinse pump 0.11 kW.

- Consumption of water per wash program: Minimum 4 litres.

- EN 1.4301 Stainless steel design: Frame, door, impeller, tanks, wash and rinse arms. Heat and noise insulation.

- Dimensions: W 1025 x D 957 x H 1736/2079 (± 25) mm. Weight: Net 257 kg - Gross 381 kg.

- Sound pressure level: 69.8 dB(A), ISO 11201

- Heat dissipation: Total 2.4 kW - Sensible 0.8 kW - Latent 1.6 kW.

- Norms and certification compliance IPX5 - CE - DIN 10534 - EN 1717 - RoHS.

Installation

- Electrical: 25 A - 11,5 kW - 3 AC 400-415 V - 50 Hz. Supplied with 2 metres long power cable to connect to an approved mains switch.

- Hot water softened inlet: 55-65 °C, 3-7 °dH (5-12 °fH) or 53-125 ppm, 1-6 bar, 15 litres/min – 3/4” male (DN 20).

- Drain: Ø 32 mm - 65 °C - 50 litres/min.

- Chemicals: 230 V built-in electrical connection box. Wash tank prepared for a detergent probe installation.

Factory fitted options

- Efficient steam condensing unit with heat recovery system. Saves up to 40% of the energy required to heat the rinse tank. Requires the high-power supply option. Connection to 5-20 °C cold water inlet. Extends the duration of the wash program up to 80 sec. Total height of the pot washing machine increased to 1964 (± 25) mm. Heat dissipation: Total: 1.3 kW - Sensible: 0.8 kW - Latent: 0.5 kW.

- Built-in unit condensing steam using cold water. Water inlet: 5-20 °C - 53-125 ppm - 3-6 bar - 15 litres/min 
- ¾” male (DN 20). Extends the duration of the wash program up to 70 sec. Heat dissipation: Total: 1.7 kW - Sensible: 0.8 kW - Latent: 0.9 kW.

- Integrated liquid detergent and rinse aid dosing dispenser with peristaltic pump. Suction hoses. Detergent dosing sensor installed in the wash tank.

- Interface for energy optimization. Relay terminal block that can be controlled by dry contact or 230 V.

- High-power supply version and other voltages - Cold or Hot water softened inlet:

  - 3 AC 400-415 V - 50 Hz - 32 A - 16.5 kW

  - 3 AC 380 V - 50 Hz - 32 A - 16.5 kW

  - 3 AC 400-415 V - 60 Hz - 32 A - 16.5 kW

  - 3 AC 380 V - 60 Hz - 32 A - 16.5 kW

- Standard power supply versions and other voltages - Hot-water softened inlet:

  - 3 AC 230 V - 50 Hz - 40 A - 11.5 kW

  - 3 AC 230 V - 60 Hz - 40 A - 11.5 kW

  - 3 AC 220 V - 60 Hz - 40 A - 11.5 kW

  - 3 AC 440 V - 60 Hz - 25 A - 11.5 kW

  - 3 AC 208 V - 60 Hz - 50 A - 11.5 kW

- Marine version: 2 level sensors in the wash tank to compensate for ship’s movement. Feet with floor anchoring system. Service access from the front.

Options

- Non-electric water softener for hot and cold water. Dual tank. 4 castors. Regeneration at 2 bar 24 litres/11 min. Salt capacity 18 kg. Dimensions: L 356 x W 356 x H 559 mm. Water inlet Ø 3/4” - 2-65 °C – 2.5-8.5 bar. Drain connection 0.375’’.

- Holder for bowls and utensils. To be placed inside the wash basket. Capacity: Up to 5-6 bowls and ladles.

- Holder for 2 GN 2/1 150 mm depth or 16 GN 1/1 grids or 8 lids GN 1/1 or 6 baking trays 600x400x20 mm. To be placed inside the wash basket.

- Pot basket. Holds 1 large pot (400 mm depth) or 2 large pots (200 mm depth). To be placed as alternative to the regular wash basket.

- Net basket with lid to wash ladles, whisks up to 500 mm length.

- Multiflex holder for saucepans, small bowls, sieves. To be placed inside the wash basket. Capacity: up to 3-5 saucepans or small bowls, sieves.

- Flexible pot holder incl. 4 rubber straps and two hooks.

- Holder to store inserts and accessories not in use. Hung at either side of the pot washing machine.

- Set of 3 scrapers to eliminate food residue before loading pots or pans into the pot washing machine.

- 10-litre canister of biodegradable granules made of rapeseed oil, calcium salt, and biodegradable polyesters.