Description

- Compact front-loaded pot and pan washing machine. High flow and pressure wash technology that operates with or without blasting of granules, which allows for reduction of time, water, energy, and detergent consumption while also eliminating the need for soaking and pre-washing cookware.

- Operating panel at eye level and within easy reach. Waterproof backlit screen with alphanumeric display. Count-down and indicator light showing program end. Preventive alerts: Change of wash water, change of Granules, time for service.

- Energy-saving mode if not used for 20 min.

- Unique QR code for online access to user manuals, servicing history, and troubleshooting guides.

- HACCP traceability via USB 2.0 port equipped with waterproof lid. 200 latest wash program parameters can be exported.

- Built-in steam condensing unit using cold water. Extends the duration of the wash program up to 60 sec.

Productivity

- 2 wash modes, granule-optimized or with water only. 6 wash programs: from 2:30 min to 6:40 min. Cleaning of wash cabin and granule collection program.

- Standard delivery: 2 Sliding inserts, 1 Granule collector, 1 Starter load of Granules 6 litres, 1 scraper, installation and user manuals, instruction poster.

Select one of the two following versions:

 Art.20715 - Flexible version to wash GN and pots - 2 GN 1/1 depth 200 mm and 2 GN 1/1 depth 150 mm.

- Hourly productivity 76 GN 1/1.

- Capacity per wash cycle: 1 GN 2/1 depth 200 mm and 2 GN 1/1 depth 150 mm - 2 GN 1/1 depth 200 mm and 2 GN 1/1 depth 150 mm or various size of pots - 2 GN 1/1 depth 150 mm and 1 big pot up to Ø 480 mm and 320 mm in height.

 Art.20716 - Gastronorm version to wash GN - 4 GN 1/1 depth 200 mm.

- Hourly productivity 76 GN 1/1.

- Capacity per wash cycle: 1 GN 2/1 depth 200 mm and 2 GN 1/1 depth 200 mm - 4 GN 1/1 depth 200 mm - 
2 GN 1/1 depth 200 mm and 1 big pot up to Ø 480 mm and 320 mm in height.

Specifications and compliance

- Wash tank 84 litres. Heating element 9 kW. Wash temperature 65 °C. 2 oscillating wash arms removable without tool. 8 polymer wash nozzles designed to optimize the lifespan of granules. High pressure pump 2.2 kW.

- Rinse tank 10.5 litres. Heating element 9 kW. Rinse temperature 85 °C. Rinse arm with 18 nozzles. Rinse pump 0.37 kW.

- Consumption of water per wash program: Minimum 4 litres.

- EN 1.4301 Stainless steel design: Frame, doors, pump and impeller, tanks, wash and rinse arms. Heat and noise insulation.

- Dimensions: W 870 x D 1017 x H 1775 (± 25) mm. Weight: Net 275 kg - Gross: 369 kg.

- Sound pressure level: 66.7 dB(A), ISO 11203

- Heat dissipation: Total 1.2 kW - Sensible 0.5 kW - Latent 0.7 kW.

- Norms and certification compliance IPX5 - CE - DIN 10534 - EN 1717 - RoHS.

Installation

- Electrical: 25 A - 12 kW - 3 AC 400-415V - 50 Hz. 2 m. long power cable to connect to an approved mains switch.

- Hot-water softened inlet: 55-65 °C - 3-7 °dH (5-12 °fH) - 53-125 PPM - 1-6 bar - 15 litres/min - ¾” male (DN 20).

- Cold-water inlet: 5-20 °C - 3-7 °dH (5-12 °fH) - 53-125 PPM - 3-6 bar - 15 litres/min - ¾” male (DN 20).

- Drain: Ø 32 mm - 65 °C - 50 litres/min.

- Chemicals: 230 V built-in electrical connection box. Wash tank prepared for a detergent probe installation.

Factory fitted options

- Efficient steam condensing unit with heat recovery system. Saves up to 35% of the energy required to heat the rinse tank. Requires the high-power supply option. Cold water inlet connection 5-20 °C - 3-7 °dH (5-12 °fH) - 
53-125 PPM - 3-6 bar - 15 litres/min - ¾” male (DN 20). Extends the duration of the wash program up to 80 sec. Total height of the pot washing machine increased to 1994 (± 25) mm. Heat dissipation: Total: 0.9 kW - 
Sensible: 0.5 kW - Latent: 0.4 kW.

- Integrated liquid detergent and rinse aid dosing dispenser with peristaltic pump. Suction hoses. Detergent dosing sensor installed in the wash tank.

- Interface for energy optimization. Relay terminal block that can be controlled by dry contact or 230 V.

- High-power supply version and other voltages - Cold or hot water softened inlet:

 3 AC 400-415 V - 50 Hz - 32 A - 17 kW

 3 AC 380 V - 50 Hz - 32 A - 17 kW

 3 AC 400-415 V - 60 Hz - 32 A - 17 kW

 3 AC 380 V - 60 Hz - 32 A - 17 kW

- Standard power supply versions and other voltages - Hot-water softened and cold-water inlet:

 3 AC 230 V - 50 Hz - 40 A - 12 kW

 3 AC 230 V - 60 Hz - 40 A - 12 kW

 3 AC 220 V - 60 Hz - 40 A - 12 kW

 3 AC 440 V - 60 Hz - 25 A - 12 kW

 3 AC 208 V - 60 Hz - 50 A - 12 kW

Options

- Non-electric water softener for hot and cold water. Dual tank. 4 castors. Regeneration at 2 bar 24 litres/11 min. Salt capacity 18 kg. Dimensions: L 356 x W 356 x H 559 mm. Water inlet Ø 3/4” - 2-65 °C - 2.5-8.5 bar. Drain connection 0.375’’.

- Short spray pipe allowing the loading on the lower sliding insert of pots or mixer bowls with a maximum dimension of Ø 480 mm, height 210 mm or a maximum height of 700 mm by removing the upper sliding insert.

- Net basket with lid to wash ladles, whisks up to 500 mm long.

- Insert for 12 lids and flat trays or grids, racks GN sized.

- Holder to store inserts and accessories not in use. Hung at either side of the pot washing machine.

- Set of 3 scrapers to eliminate food residue before loading pots or pans into the pot washing machine.

- 10-litre canister of biodegradable granules made of rapeseed oil and biodegradable polyesters.